## 1. Simplification of Boolean Function Using K-Map Method (Up to 5 Variables)

A Karnaugh Map (K-map) is a graphical method for minimizing Boolean expressions without using Boolean algebra. It is based on the principle of adjacency and grouping of minterms that differ by only one variable. K-maps reduce a Boolean function to its simplest Sum-of-Products (SOP) or Product-of-Sums (POS) form.

### Key Points:

- K-maps exist for 2, 3, 4, and 5 variables.

- Adjacent cells differ by only one bit (Gray code arrangement).

- Groups must be in powers of 2: 1, 2, 4, 8, 16, 32.

- Larger groups give simpler expressions.

- Valid groups include wrapping around edges and corners.

- Prime Implicants are all possible largest groups.

- Essential Prime Implicants cover minterms not covered by any other implicant.

### Procedure:

- Draw appropriate K-map (2ⁿ cells).

- Fill 1s for minterms (or 0s for maxterms).

- Identify all possible groups of adjacent 1s.

- Write simplified expression by eliminating variables that remain constant in a group.

- Combine all essential groups to get minimal expression.

### Advantages:

- Visual and intuitive.

- Guarantees minimal expression for ≤5 variables.

- Useful for designing minimal combinational circuits.

## 2. Simplification Using Quine-McCluskey Method

The Quine-McCluskey method is a tabular minimization technique suitable for computer implementation and capable of handling many variables.

### Key Points:

- Systematic, deterministic, unlike graphical K-maps.

- Works for SOP or POS forms.

- Produces Prime Implicant Table.

### Procedure:

- Group minterms based on the number of 1s in binary representation.

- Combine minterms differing in exactly one bit by replacing differing bit with ‘–’ (don’t care).

- Repeat combinations until no more combinations are possible.

- The remaining terms are Prime Implicants.

- Form the Prime Implicant Chart.

- Select Essential Prime Implicants and other necessary implicants using minimum cover method.

### Advantages:

- Suitable for digital logic CAD tools.

- Works for many variables (≥6).

- Provides exact minimal solutions.

## 3. NAND and NOR Implementation

NAND and NOR gates are universal gates, meaning any Boolean function can be realized using only NANDs or only NORs.

### Key Concepts:

- NAND as Universal Gate:

  - NOT A = A NAND A

  - A AND B = (A NAND B) NAND (A NAND B)

  - A OR B = (A NAND A) NAND (B NAND B)

- NOR as Universal Gate:

  - NOT A = A NOR A

  - A OR B = (A NOR B) NOR (A NOR B)

  - A AND B = (A NOR A) NOR (B NOR B)

### Implementation Steps:

- Optimize Boolean expression first.

- Convert to NAND-NAND or NOR-NOR form.

- Replace all operations using universal gate equivalents.

- Draw final logic diagram.

### Advantages:

- Cost-effective hardware.

- Simpler IC libraries.

- Supports standard PLA/PAL design.

## 4. Combinational Logic: Introduction

Combinational Logic Circuits are circuits whose output depends solely on the present input values, without any memory element.

### Features:

- No feedback or clock signal.

- Output = f(inputs).

- Implemented using logic gates.

- Used in arithmetic units, data routing, and control systems.

### General Types:

- Arithmetic circuits

- Encoding/decoding circuits

- Multiplexing circuits

- Code converters

## 5. Analysis and Design Procedure of Combinational Logic

### Analysis Procedure:

- Identify all inputs and outputs.

- Derive truth table.

- Obtain Boolean expressions.

- Simplify expression using algebra/K-map/Quine-McCluskey.

- Draw logic diagram.

### Design Procedure:

- Determine problem specifications.

- Define input and output variables.

- Construct truth table according to specification.

- Derive minimized Boolean functions.

- Implement function using logic gates.

- Verify circuit operation.

## 6. Binary Adder and Subtractor

### Half Adder:

- Adds two bits (A, B).

- Sum = A ⊕ B

- Carry = A · B

### Full Adder:

- Adds A, B, and Cin.

- Sum = A ⊕ B ⊕ Cin

- Cout = AB + BCin + ACin

### Binary Subtractor:

- Uses 2’s complement or borrow logic.

- Full subtractor outputs:

  - Difference = A ⊕ B ⊕ Bin

  - Bout = B·Bin + B·A′ + Bin·A′

### Adder-Subtractor Combined Circuit:

- Uses XOR gates with a mode control.

- When M = 0 → add

- When M = 1 → subtract (B ⊕ 1 = B′, Cin = 1)

## 7. Decimal Adder

A decimal adder adds two BCD digits and produces a BCD result.

### Key Points:

- Uses binary addition plus correction.

- If sum > 9 or carry = 1, add 0110 (6) for correction.

- Requires 4-bit adder + correction circuit.

### Working:

- Add BCD digits using binary adder.

- Check for invalid BCD results (1010–1111).

- Add 6 to adjust to valid BCD.

- Generate carry for next digit.

## 8. Binary Multiplier

Binary multiplication is repeated addition of shifted multiplicands.

### Hardware Implementation:

- AND gates for partial product generation.

- Adders for summing partial products.

- Structure similar to manual multiplication.

### Types:

- Array Multiplier

- Booth Multiplier (signed numbers)

- Sequential Multiplier

## 9. Magnitude Comparator

A comparator compares two binary numbers A and B.

### Outputs:

- A > B

- A = B

- A < B

### Logic:

- Compare bits from MSB to LSB.

- Use combinational logic for each condition.

### Typical IC:

- 7485 4-bit comparator.

## 10. Decoders

A decoder converts n input lines into a maximum of 2ⁿ output lines.

### Examples:

- 2-to-4 decoder

- 3-to-8 decoder

- 4-to-16 decoder

### Applications:

- Memory addressing

- Instruction decoding

- Data selection

## 11. Encoders

An encoder performs the reverse operation of a decoder, converting 2ⁿ distinct inputs into an n-bit output.

### Types:

- Simple encoder

- Priority encoder (resolves multiple inputs)

## 12. Multiplexers (MUX)

A multiplexer selects one input from many data lines and transmits it to the output based on select lines.

### Example:

- 4:1, 8:1, 16:1 MUX.

### Boolean function implementation:

Any Boolean function can be implemented using a MUX.

## 13. Demultiplexers (DEMUX)

A DEMUX takes a single input and routes it to one of many output lines depending on select inputs.

### Applications:

- Data distribution

- Communication systems

- Timing circuits

## 14. Code Conversion

Used to convert data from one binary code to another.

### Common Conversions:

- BCD to Excess-3

- Excess-3 to BCD

- Gray to Binary

- Binary to Gray

### Approach:

- Construct truth table.

- Minimize outputs using K-map.

- Implement using logic gates.

## 15. Introduction to HDL Description of Combinational Logic Circuits

HDL (Hardware Description Language) such as Verilog or VHDL is used to describe hardware at behavioral, dataflow, and structural levels.

### Key Concepts:

- Behavioral modeling: uses always/if statements.

- Dataflow modeling: uses continuous assignments.

- Structural modeling: gate-level interconnection.

### Advantages:

- Fast prototyping.

- Simulation before fabrication.

- Reusable code.

- Supports synthesis to digital hardware.

### Basic HDL Structure for Combinational Logic:

- Module/entity declaration

- Input/output ports

- Continuous assignments (assign in Verilog)

- No clock or memory elements

# 1. Introduction to Sequential Logic

Sequential logic circuits are digital circuits whose output depends on both current inputs and previous states. Unlike combinational circuits, they contain memory elements such as latches, flip-flops, registers, and counters.

### Key Features

- Output = f(inputs, present state)

- Contains feedback loops

- Requires clock (in synchronous circuits)

- Can store binary information

- Forms the basis of registers, counters, state machines

### Components:

- Combinational logic

- Memory elements

- Clock (for synchronous circuits)

### Applications:

- CPUs and controllers

- Counters and registers

- Communication systems

- Digital control systems

# 2. Types of Sequential Circuits

Sequential circuits are broadly classified into:

## (A) Synchronous Sequential Circuits

- All memory elements are triggered by a common clock signal.

- Operation occurs at discrete intervals.

- Easy to design and analyze.

- Examples: Synchronous counters, shift registers.

## (B) Asynchronous Sequential Circuits

- No clock signal; change depends on input change and propagation delay.

- Faster but prone to hazards and race conditions.

- Used in small and simple control circuits.

### Comparison

| Feature | Synchronous | Asynchronous |
| --- | --- | --- |
| Triggering | Clock-based | Input change |
| Speed | Slower | Faster |
| Stability | Very stable | Can be unstable |
| Complexity | Easy to design | Harder |
| Example | Registers, Counters | Simple control circuits |

# 3. Basic Storage Elements (Latches and Flip-Flops)

Memory in sequential circuits is implemented using simple bistable devices.

## (A) Latches

Latches are level-triggered storage elements.

### Types of Latches:

### 1. SR Latch (Set-Reset)

- Constructed using NOR or NAND gates.

- Inputs: S (Set), R (Reset)

- When S=1 → Q=1

- When R=1 → Q=0

- Invalid state when S=1 and R=1 (NOR latch)

### 2. D Latch

- Eliminates invalid condition of SR latch.

- D latch passes input D to output when Enable=1.

- When Enable=0, it holds the last value.

## (B) Flip-Flops

Flip-flops are edge-triggered devices (positive or negative edge).

### 1. SR Flip-Flop

Same as SR latch but triggered on clock edge.

### 2. D Flip-Flop

- Also known as Data or Delay FF.

- On clock edge, Q = D.

- Used in registers and memories.

### 3. JK Flip-Flop

- Modified SR FF where J=K=1 is allowed.

- J=1, K=0 → Set

- J=0, K=1 → Reset

- J=1, K=1 → Toggle output

### 4. T Flip-Flop

- Toggles output on every clock pulse when T = 1.

- Used for counters.

# 4. Characteristic Equations & Characteristic Tables

Characteristic equation represents the next state (Qₙ₊₁) in terms of present state (Qₙ) and inputs.

### (A) SR Flip-Flop

- Characteristic Table:

| S | R | Qₙ₊₁ |
| --- | --- | --- |
| 0 | 0 | Qₙ |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Invalid |

- Characteristic Equation:
Qₙ₊₁ = S + R′Qₙ

### (B) D Flip-Flop

| D | Qₙ₊₁ |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

- Characteristic Equation:
Qₙ₊₁ = D

### (C) JK Flip-Flop

| J | K | Qₙ₊₁ |
| --- | --- | --- |
| 0 | 0 | Qₙ |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Q̅ₙ |

- Characteristic Equation:
Qₙ₊₁ = JQ̅ₙ + K′Qₙ

### (D) T Flip-Flop

| T | Qₙ₊₁ |
| --- | --- |
| 0 | Qₙ |
| 1 | Q̅ₙ |

- Characteristic Equation:
Qₙ₊₁ = TQ̅ₙ + T′Qₙ

# 5. Excitation Tables

Excitation table shows what inputs are needed to achieve a desired transition.

### (A) SR Flip-Flop

| Qₙ → Qₙ₊₁ | S | R |
| --- | --- | --- |
| 0 → 0 | 0 | 0 |
| 0 → 1 | 1 | 0 |
| 1 → 0 | 0 | 1 |
| 1 → 1 | 0 | 0 |

### (B) D Flip-Flop

| Qₙ → Qₙ₊₁ | D |
| --- | --- |
| 0 → 0 | 0 |
| 0 → 1 | 1 |
| 1 → 0 | 0 |
| 1 → 1 | 1 |

### (C) JK Flip-Flop

| Qₙ → Qₙ₊₁ | J | K |
| --- | --- | --- |
| 0 → 0 | 0 | 0 |
| 0 → 1 | 1 | 0 |
| 1 → 0 | 0 | 1 |
| 1 → 1 | 0 | 0 or 1 1 |

### (D) T Flip-Flop

| Qₙ → Qₙ₊₁ | T |
| --- | --- |
| 0 → 0 | 0 |
| 0 → 1 | 1 |
| 1 → 0 | 1 |
| 1 → 1 | 0 |

# 6. Flip-Flop Conversion

Used to convert one flip-flop type into another by justifying excitation values.

### Procedure:

- Write the characteristic table of desired FF.

- Write excitation table of given FF.

- Compare transitions.

- Draw K-maps for input functions required.

- Derive expressions and draw circuit.

### Common Conversions:

- SR → JK

- JK → T

- D → JK

- JK → D

Used in designing counters and controlled sequential circuits.

# 7. Analysis of Synchronous Sequential Circuits

### Steps:

- Identify flip-flops and label states.

- Obtain input equations to FFs.

- Use characteristic equations of the FFs.

- Form state table (Present State → Next State).

- Derive output table (Mealy or Moore).

- Draw state diagrams.

- Simplify using K-map if required.

### Outputs:

- Mealy Machine: Output depends on state & input.

- Moore Machine: Output depends only on state.

# 8. Design of Synchronous Sequential Circuits

The design procedure creates circuits like counters, registers, and control units.

### Steps:

- Define the problem (e.g., sequence detector).

- List all possible states and assign binary codes.

- Create state transition table.

- Choose flip-flop type (usually D or JK).

- Use excitation table to find required inputs.

- Obtain simplified logic expressions via K-map.

- Draw the final circuit.

### Applications:

- Sequence detectors

- Pattern recognizers

- Synchronous counters

- Moore/Mealy machines used in CPUs and controllers

# 1. Registers – Introduction

A register is a group of flip-flops used to store a multi-bit binary value. If one flip-flop stores 1 bit, an n-bit register contains n flip-flops. Registers are fundamental storage elements inside the CPU and digital systems.

### Functions

- Temporary data storage

- Holding intermediate results

- Data transfer between ALU and memory

- Used in counters, shift operations, and control units

### Types

- Buffer Register – temporary data

- Program Counter (PC)

- Instruction Register (IR)

- Accumulator (AC)

- General-purpose registers

# 2. Shift Register

Shift registers store data and shift it left or right under clock control.

### Operation Modes

- Serial-In Serial-Out (SISO)

- Serial-In Parallel-Out (SIPO)

- Parallel-In Serial-Out (PISO)

- Parallel-In Parallel-Out (PIPO)

### Applications

- Serial–parallel data conversion

- Delay elements

- Ring counters

- Communication interface

- Arithmetic operations (multiplication/division by 2)

# 3. Universal Shift Register

A universal shift register can perform all possible shift operations:

### Features

- Shift left

- Shift right

- Parallel load

- Hold operation

### Mode Control Table (Typical)

| Mode | Operation |
| --- | --- |
| 00 | Hold |
| 01 | Shift Right |
| 10 | Shift Left |
| 11 | Parallel Load |

### Applications

- CPU data path

- Arithmetic logic blocks

- Register-to-register transfer

- Serial communication

# 4. Counters – Introduction

A counter is a sequential circuit that goes through a predefined sequence of states.

### Types

- Ripple (Asynchronous) Counter

- Synchronous Counter

- Up/Down counters

- Ring counter

- Johnson counter

# 5. Ripple Counter (Asynchronous Counter)

In ripple counters, the flip-flops do not share a common clock.
The output of one flip-flop acts as the clock input to the next.

### Characteristics

- Simple design

- Easy to implement

- Speed limited due to cumulative propagation delay

- Example: 4-bit ripple counter counts 0–15

### Design

- Use T flip-flops or JK flip-flops in toggle mode (T=1)

# 6. Synchronous Counter

In synchronous counters, all flip-flops receive the same clock simultaneously.

### Features

- Faster due to no ripple delay

- All FFs triggered together

- More complex logic

- Can implement up/down, decade, and presettable counters

### Design Steps

- Write state sequence

- Choose flip-flop type

- Prepare excitation table

- Use K-maps to derive FF input logic

- Draw the final circuit

# 7. Introduction to Memory

Memory stores binary information required for computation.

### Classification

## (A) Primary Memory

- RAM (Random Access Memory)

  - Volatile

  - Types: SRAM, DRAM

- ROM (Read-Only Memory)

  - Non-volatile

  - Types: PROM, EPROM, EEPROM

## (B) Secondary Memory

- HDD, SSD, Pen drives

## (C) Cache Memory

- High-speed memory between CPU and RAM

- L1, L2, L3 caches

## (D) Virtual Memory

- Logical extension of RAM using disk

# 8. Types of Memory

### (1) SRAM

- Uses flip-flops

- Faster, expensive

- Used in cache

### (2) DRAM

- Uses capacitor-based cells

- Slower, cheaper

- Used in main memory

### (3) PROM

- Programmable once

- Uses fusible links

### (4) EPROM

- Erasable using UV light

### (5) EEPROM

- Electrically erasable, reusable

### (6) Flash Memory

- Type of EEPROM

- Used in SSD, USB drives

# 9. Programmable Logic Devices (PLDs): PAL, PLA, ROM

PLDs allow engineers to implement digital circuits in programmable form.

## A. PAL (Programmable Array Logic)

- Programmable AND array

- Fixed OR array

- Faster, simpler

- Limited flexibility

## B. PLA (Programmable Logic Array)

- Programmable AND + Programmable OR arrays

- More flexible than PAL

- Supports complex logic with custom OR terms

- Used in custom combinational logic design

## C. ROM (Read-Only Memory)

- Fixed connections in AND plane

- Programmable OR plane

- Stores truth tables directly

- Used for lookup tables & microprogramming

# ⭐ Computer Organization & Architecture Topics

# 10. Introduction to Computer Organization & Architecture

### Computer Organization

- Describes hardware structure

- Includes CPU design, memory organization, I/O system

- Focuses on implementation of computer components

### Computer Architecture

- Describes functional behavior of computer

- Includes instruction set, addressing modes, data formats

- Visible to programmer

### Difference

- Architecture → What computer does

- Organization → How it does it

# 11. Von Neumann and Harvard Architecture

## A. Von Neumann Architecture

- Single memory for data & instructions

- Single bus system

- Sequential instruction execution

- Simpler, cheaper

- Used in general-purpose computers

### Limitation:

Von Neumann Bottleneck: Shared bus delays execution.

## B. Harvard Architecture

- Separate memory for program & data

- Two separate buses

- Allows parallel access

- Faster execution

### Used In

- DSP processors

- Microcontrollers (PIC, AVR)

# 12. RISC and CISC Machines

## RISC (Reduced Instruction Set Computer)

- Simple, fixed-length instructions

- Load–store architecture

- Hardwired control

- Executes one instruction per cycle

- Easier pipelining

- Example: ARM, MIPS, RISC-V

## CISC (Complex Instruction Set Computer)

- Large, complex instruction set

- Variable length instructions

- Microprogrammed control

- Fewer lines of assembly needed

- Example: Intel x86

# 13. Evolution of Intel x86 and ARM Architecture

## Intel x86

- Started from 8086 (16-bit)

- Evolved to 32-bit (80386)

- Modern x86-64 supports 64-bit processing

- Used in desktops, laptops, servers

- Emphasizes high performance, backward compatibility

## ARM Architecture

- Developed for low power & embedded systems

- RISC-based

- Used in smartphones, tablets, IoT

- Modern versions (ARMv8, ARMv9) provide 64-bit support

- Prioritizes energy efficiency

# 14. Basic Measures of Computer Performance

### (1) Response Time

Time taken to complete a task.

### (2) Throughput

Number of tasks completed per unit time.

### (3) CPU Time

CPU time = IC × CPI × Clock Cycle Time
Where:

- IC = Instruction Count

- CPI = Cycles per Instruction

### (4) MIPS & MFLOPS

- MIPS → Millions of Instructions per Second

- MFLOPS → Millions of Floating Point Operations per Second

# 15. Amdahl’s Law

Defines theoretical speedup of a program by improving one of its components.

### Formula:

Speedup=1(1−P)+PS\text{Speedup} = \frac{1}{(1 - P) + \frac{P}{S}}Speedup=(1−P)+SP​1​

Where:

- P = fraction of program improved

- S = speedup of improved part

### Meaning:

Performance improvement is limited by the part that cannot be improved.

# 16. Little’s Law

Used in analyzing queues in computer systems.

### Formula:

L=λWL = \lambda WL=λW

Where:

- L = average number of items in system

- λ = arrival rate

- W = average waiting time

### Applications:

- CPU scheduling

- Network traffic

- Memory and cache analysis

## 1. Integer Representation

### Definition

Integer representation defines how positive and negative whole numbers are stored in binary form inside digital systems.

### Types of Integer Representation

#### (a) Sign-Magnitude Representation

- MSB (Most Significant Bit) is the sign bit.

- 0 → positive, 1 → negative

- Remaining bits represent magnitude.

Example:
+7 = 0 111
−7 = 1 111

Advantage: Easy sign interpretation
Disadvantage: Two representations of zero (0000 and 1000)

#### (b) 1’s Complement

- Negative numbers obtained by complementing all bits of positive number.

Example:
+5 = 0101
−5 = 1010

Problems: Two zeros (0000 and 1111)

#### (c) 2’s Complement (Most widely used)

- Take 1’s complement and add 1.

Example:
+5 = 0101
1’s complement = 1010
Add 1 → 1011 = −5

Advantages:

- Single zero

- Easy arithmetic

- Used in all CPUs

## 2. Integer Arithmetic

### (a) Addition

Rule:
Add normally in binary; discard carry if overflow in signed addition.

Example:
5 + (−3)
5 → 0101
−3 → 1101 (2’s complement)
Add: 0101
1101
= 10010 → discard carry → 0010 = 2

### (b) Subtraction

Rule:
A − B = A + (2’s complement of B)

Example:
7 − 5
5 (2’s comp) = 1011
Add: 0111
1011
= 10010 → discard carry → 0010 = 2

### (c) Multiplication

Performed using repeated addition or shift-and-add algorithm.

Example: 6 × 3
6 = 0110
3 = 0011

Step 1: Last bit of 3 is 1 → Add 6
Step 2: Shift 6 left
Step 3: Next bit 1 → Add

Result = 18 = 10010

### (d) Division

Binary division is repeated subtraction + shifting.

Example:
20 ÷ 4
10100 ÷ 0100 = 101 (5)

# ⭐ Floating-Point Representation (IEEE-754 Standard)

A real number is stored as:

Number = (−1)^Sign × Mantissa × 2^(Exponent − Bias)

### IEEE-754 Single Precision (32 bits)

- Sign: 1 bit

- Exponent: 8 bits

- Mantissa: 23 bits

- Bias = 127

### Example: Represent +5.75 in IEEE-754

- Convert to binary:
5.75 = 101.11

- Normalize: 1.0111 × 2²

- Sign = 0

- Exponent = 2 + 127 = 129 = 10000001

- Mantissa = 0111……

Final 32-bit representation:
0 10000001 01110000000000000000000

## Floating-Point Addition (Steps)

- Equalize exponents

- Add/sub mantissas

- Normalize

- Round

# ⭐ Machine Instruction Characteristics

A machine instruction is the binary code executed by a CPU.

### Characteristics of Instructions

- Operation (Opcode) – defines action (ADD, LOAD).

- Operands – define data or memory location.

- Instruction Size – fixed or variable length.

- Instruction Format – R-type, I-type, J-type.

- Processor Compatibility – must match architecture.

Example:
ADD R1, R2, R3
→ R1 = R2 + R3

# ⭐ Addressing Modes

Techniques to locate operands.

### 1. Immediate Mode

Operand is inside instruction.
Example: MOV R1, #5

### 2. Register Mode

Operand stored in CPU register.
Example: ADD R1, R2

### 3. Direct Addressing

Instruction contains memory address.
Example: LOAD R1, 500

### 4. Indirect Addressing

Instruction contains address of a memory location containing actual address.
Example: LOAD R1, (500)

### 5. Indexed Mode

Address = Base register + constant
Used in arrays.

# ⭐ Processor Structure & Operation

A processor has:

### 1. ALU

Performs arithmetic and logic.

### 2. Registers

Fastest memory.
Types: General purpose, Special purpose.

### 3. Control Unit (CU)

Fetches, decodes, and executes instructions.

### 4. Data Path

Movement of data across components.

# ⭐ Instruction Cycle

- Fetch – Get instruction from memory

- Decode – CU interprets

- Execute – ALU performs

- Memory Access – Load/Store

- Write Back – Result saved

- PC update – Next instruction

Diagram you can draw in exam

Fetch → Decode → Execute → Memory → Write-back

# ⭐ Instruction Pipelining

### Definition

A technique in which multiple instructions are processed simultaneously in overlapping phases.

### Stages (5-Stage RISC pipeline)

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write Back

### Performance

Speedup ≈ Number of stages
Pipeline throughput increases but latency per instruction remains same.

### Hazards in Pipeline

- Structural Hazards – hardware conflict

- Data Hazards – dependency between instructions

- Control Hazards – branching affects pipeline

Example:
ADD R1, R2, R3
SUB R4, R1, R5 → Data hazard (R1 dependent)

# ⭐ Control Unit Operation

### Types of Control Units

## 1. Hardwired Control Unit

- Control signals generated by combinational logic.

- Fast but difficult to modify.

Used in: RISC processors.

## 2. Microprogrammed Control Unit

- Uses control memory to store micro-instructions.

- Easy to design and modify.

Used in: CISC processors.

### Microprogrammed Control Working

- Microinstructions define sequence of operations.

- CU reads microinstruction → generates control signals.

Example:
ADD instruction may take:
Microstep1: Fetch operand
Microstep2: Add in ALU
Microstep3: Store result

Definition

Memory hierarchy is the arrangement of storage devices based on speed, cost, and capacity, providing the illusion of a large, fast, and low-cost memory to the CPU.

Structure of Memory Hierarchy (Top to Bottom)

- Registers – Fastest, smallest, most expensive

- Cache Memory

- Main Memory (RAM)

- Secondary Memory (SSD/HDD)

- Tertiary Storage (Optical, Tape)

Diagram (describe in exam):
A pyramid with Register → Cache → Main Memory → Disk → Tape
Speed ↓, Cost ↓, Capacity ↑

⭐ Locality and Performance

Locality of Reference

Programs tend to access a small portion of memory repeatedly.

Types of Locality

- Temporal Locality

  - Recently accessed data will be accessed again soon.
Example: Loop variables, counters.

- Spatial Locality

  - Nearby memory locations are likely to be accessed soon.
Example: Sequential code execution, array traversal.

- Sequential Locality

  - Instructions are fetched in sequence.

Impact on Performance

- Locality increases cache hit rate.

- High cache hit rate = faster CPU performance.

⭐ Cache Memory

Definition

Cache memory is a small, fast memory placed between CPU and main memory to reduce memory access time.

⭐ Principles of Cache Design

1. Cache Mapping Techniques

(a) Direct Mapping

- Each memory block maps to exactly one cache line.

- Simple, fast, but suffers collisions.

Formula:
Cache Line = (Memory Block) mod (Number of Lines)

(b) Fully Associative Mapping

- Any memory block can go to any cache line.

- Highest flexibility, but expensive.

(c) Set-Associative Mapping

- Cache divided into sets; each set has multiple lines.

- Combines benefits of direct and associative mapping.

- Example: 4-way set associative

2. Cache Replacement Policies

- LRU (Least Recently Used)

- FIFO (First In First Out)

- Random Replacement

3. Cache Write Policies

Write-Through

Write to both cache and memory → simple but slower.

Write-Back

Write only to cache; memory updated later → faster.

⭐ Elements of Cache Design

- Cache size

- Block size

- Associativity

- Replacement policy

- Write policy

- Hit and miss handling circuitry

⭐ Internal Memory

Memory inside the computer:

1. RAM (Random Access Memory)

- Static RAM (SRAM) → used in cache

- Dynamic RAM (DRAM) → used in main memory

2. ROM (Read Only Memory)

- Stores firmware

- Types: PROM, EPROM, EEPROM

3. Flash Memory

- Faster than HDD

- Used in SSDs and embedded systems

⭐ External Memory

Used for long-term storage:

1. Hard Disk Drives (HDD)

- Magnetic storage

- Large capacity but slower

2. Solid State Drives (SSD)

- Flash-based

- Faster, shock resistant

3. Optical Storage

- CD/DVD/Blu-ray

⭐ I/O INTERFACE

Purpose

The I/O interface manages data exchange between CPU and external devices like keyboards, displays, disks, and networks.

⭐ External Devices

Two main categories:

1. Human Readable Devices

Keyboard, Monitor, Printer

2. Machine Readable Devices

Disk drives, Sensors, Network cards

⭐ I/O Modules

Definition

Hardware units controlling communication between CPU and external devices.

Functions of I/O modules

- Command decoding

- Data buffering

- Error detection

- Device control

Example:
USB controller, PCI controller, Disk controller

⭐ Programmed I/O

Definition

CPU performs all I/O operations by executing instructions.

Working

- CPU checks device status

- Waits until device is ready

- CPU transfers each byte

Disadvantage

CPU spends time waiting → inefficient.

Use case

Simple or slow devices.

⭐ Interrupt Driven I/O

Definition

CPU performs other tasks and is interrupted only when I/O device is ready.

Working

- CPU issues command

- CPU continues execution

- Device sends interrupt

- CPU handles interrupt and transfers data

Advantage

Efficient CPU utilization.

⭐ Direct Memory Access (DMA)

Definition

DMA controller transfers data directly between I/O device and memory without CPU involvement.

Working

- CPU initializes DMA with memory address

- DMA transfers data

- DMA sends interrupt after completion

Advantages

- High-speed data transfer

- CPU free during transfer

Used in:
Disk drives, sound cards, network cards.

⭐ Introduction to Alternative Architectures

1. SISD (Single Instruction, Single Data)

Traditional sequential computers.

2. SIMD (Single Instruction, Multiple Data)

Perform same operation on multiple data.
Used in: vector processors, GPUs.

3. MIMD (Multiple Instruction, Multiple Data)

Multiple processors executing different instructions.
Used in: modern multicore systems.

4. Multicore Architectures

Multiple processing cores on same chip → high parallelism.

5. GPU Architecture

Hundreds/thousands of ALUs for parallel computing.

⭐ SUMMARY TABLE FOR REVISION

| Topic | Key Points |
| --- | --- |
| Memory Hierarchy | Registers → Cache → RAM → Disk |
| Locality | Temporal, spatial, sequential |
| Cache | Direct, associative, set-associative |
| Write Policies | Write-through, write-back |
| Internal Memory | SRAM, DRAM, ROM |
| External Memory | HDD, SSD, Optical |
| I/O Interface | Manages CPU–device communication |
| Programmed I/O | CPU waits |
| Interrupt I/O | CPU interrupted when needed |
| DMA | High-speed transfer without CPU |
| Alternative Architectures | SISD, SIMD, MIMD, Multicore, GPU |